# CHAPTER 1

# Your First Engineering Program: Solving a Quadratic Equation with MATLAB and AI

## Learning Objectives

After completing this chapter, you should be able to:

- translate a mathematical problem into computational steps;

- develop a simple flowchart;

- write and run a MATLAB script;

- use variables, operators, and built-in functions;

- use if–elseif–else for logical decisions;

- solve and visualize a quadratic equation;

- use MATLAB's roots() function;

- use AI to improve a program; and

- verify and interpret computational results as an engineer.

# 1.1 Programming in the AI Era

AI can generate MATLAB code within seconds.

MATLAB itself already contains thousands of built-in functions.

So why should an engineering student still learn programming?

Because the engineer must still:

- understand the problem;

- formulate the mathematics;

- develop or evaluate the computational logic;

- verify the result;

- interpret its physical meaning; and

- make the engineering decision.

AI can write code. The engineer remains responsible for the solution.

Throughout this book, we will follow:

PROBLEM → THINK → BUILD → RUN → AI → VERIFY → INTERPRET

We will not learn MATLAB by memorizing commands one by one.

We will start with problems.

# 1.2 The First Problem: A Quadratic Equation

Consider

ax2+bx+c=0

with the familiar solution

x1,2=-b±b2-4ac2a

Start with

x2-5x+6=0

so that

a=1, b=-5, c=6.

Before opening MATLAB, identify:

### Input

a, b, c

### Process

Calculate the discriminant

D=b2-4ac

and then the roots.

### Output

x1, x2

We have already developed the basic algorithm.

Good programming begins with thinking, not typing.

# 1.3 From Algorithm to Flowchart

A flowchart describes the logic of a program before the code is written.

Standard symbols are:

| Symbol | Meaning |
| --- | --- |
| Ellipse | Start / End |
| Parallelogram | Input / Output |
| Rectangle | Process |
| Diamond | Decision |
| Arrow | Direction of flow |

For the quadratic equation, the logic is:

START

↓

Input a,b,c

↓

◇ Is a=0?

YES → Not a quadratic equation

NO ↓

Calculate

D=b2-4ac

↓

◇ Is D>0?

YES → Two distinct real roots

NO ↓

◇ Is D=0?

YES → One repeated real root

NO → Two complex conjugate roots

↓

Display solution

↓

END

### Think

Why must we check

a=0

before using the quadratic formula?

# 1.4 Build the Simplest MATLAB Program

Open MATLAB and create a script.

a = 1;

b = -5;

c = 6;

D = b^2 - 4*a*c;

x1 = (-b + sqrt(D))/(2*a);

x2 = (-b - sqrt(D))/(2*a);

disp(x1)

disp(x2)

Run the program.

You should obtain

x1=3, x2=2.

You have now written a MATLAB program.

### What did we use?

| MATLAB | Meaning |
| --- | --- |
| a = 1 | Variable and assignment |
| + - * / ^ | Arithmetic operators |
| sqrt() | Built-in function |
| disp() | Display output |
| ; | Suppress automatic display |

### Experiment

Remove the semicolon from

a = 1;

Run the program again.

What changes?

# 1.5 Experiment, Predict, Verify

Change the equation to

x2-7x+12=0.

Change only:

a = 1;

b = -7;

c = 12;

Before running MATLAB:

### Predict

What should the roots be?

### Run

Execute the program.

### Verify

Substitute the calculated roots back into

x2-7x+12.

Do you obtain zero?

Prediction before computation and verification after computation are good engineering habits.

# 1.6 Let the Program Make Decisions

The discriminant

D=b2-4ac

determines the nature of the roots:

D>0

→ two distinct real roots

D=0

→ one repeated real root

D<0

→ two complex conjugate roots.

MATLAB can represent this logic using:

if

...

elseif

...

else

...

end

A compact flowchart-based program is:

a = input('Enter a: ');

b = input('Enter b: ');

c = input('Enter c: ');

if a == 0

disp('This is not a quadratic equation.')

else

D = b^2 - 4*a*c;

if D > 0

x1 = (-b + sqrt(D))/(2*a);

x2 = (-b - sqrt(D))/(2*a);

fprintf('Two real roots: %.3f and %.3f\n',x1,x2)

elseif D == 0

x = -b/(2*a);

fprintf('One repeated root: %.3f\n',x)

else

x1 = (-b + sqrt(D))/(2*a);

x2 = (-b - sqrt(D))/(2*a);

disp('Two complex roots:')

disp([x1 x2])

end

end

Now the computer does more than calculate.

It follows logic.

# 1.7 MATLAB Already Has a Shortcut

MATLAB has a built-in function:

roots()

For

x2-5x+6=0,

use:

x = roots([1 -5 6])

or:

a = 1;

b = -5;

c = 6;

x = roots([a b c])

Compare the answer with your own algorithm.

They should agree.

### Important Question

If MATLAB can solve the equation using one command, why did we write our own program?

Because:

Built-in functions give us speed. Building the algorithm gives us understanding.

A good engineer needs both.

# 1.8 Visualize the Solution

Consider

y=x2-5x+6.

Plot it:

x = -2:0.1:7;

y = x.^2 - 5*x + 6;

plot(x,y)

grid on

xlabel('x')

ylabel('y')

title('Quadratic Function')

Look at the graph.

Where does the curve cross the x-axis?

Compare these locations with

x=2, x=3.

The analytical solution, MATLAB calculation, and graph should tell the same story.

# 1.9 From Mathematics to Civil Engineering

Quadratic equations can appear in engineering problems involving:

- trajectories;

- geometry;

- structures;

- hydraulics;

- optimization; and

- intersections of physical relationships.

Consider a simple example.

## Object Moving Above a Water Surface

An object is projected upward from a bridge deck located

20 m

above the water surface.

Its initial upward velocity is

v0=10 m/s.

The vertical position is

z=z0+v0t-12gt2

where

g=9.81 m/s2.

When the object reaches the water surface,

z=0.

Therefore,

0=20+10t-129.81t2

or

4.905t2-10t-20=0.

Use:

a = 4.905;

b = -10;

c = -20;

t = roots([a b c])

MATLAB produces two mathematical roots.

### Think

- What does trepresent?

- Which root represents the event after t=0?

- Does a negative time have physical meaning in this problem?

This illustrates an important principle:

A mathematically correct solution is not necessarily a physically meaningful engineering solution.

MATLAB solves the mathematics.

The engineer interprets the physics.

# 1.10 Now Bring in AI

We first developed the logic ourselves.

Now let AI help improve the program.

Try a prompt such as:

Improve this MATLAB quadratic-equation program. Add user input, error checking, classification of the roots, a graph of the function, the roots, and the vertex. Explain the major improvements.

Then examine the AI-generated code.

Do not simply copy it.

Ask:

- What did AI change?

- Why?

- What new MATLAB commands did it introduce?

- Do you understand them?

- Are the answers correct?

- Is the new program actually better?

- Does the graph agree with the calculated roots?

AI-generated code must be examined and verified.

# 1.11 Four Ways to Solve the Problem

We now have four approaches.

### 1. Mathematics

x1,2=-b±b2-4ac2a

### 2. Our MATLAB Algorithm

Program the calculation and logic ourselves.

### 3. MATLAB Built-In Function

roots([a b c])

### 4. AI-Assisted MATLAB

Ask AI to generate, explain, debug, or improve the program.

Which approach is best?

It depends on the problem.

The engineer must know when to use each one.

# 1.12 Think Like an Engineer

## Analyze

Why does the discriminant determine the nature of the roots?

## Evaluate

Which would you trust more?

Program A:
15 lines of MATLAB that you completely understand.

Program B:
100 lines generated by AI that you only partly understand.

Explain your answer.

## Create

Develop a quadratic-equation program that:

- accepts a,b,c;

- identifies the type of roots;

- calculates the roots;

- plots the function; and

- marks the roots on the graph.

You may use AI.

But you must be able to:

explain, verify, and defend your solution.

# 1.13 Chapter Takeaway

From one quadratic equation, we have encountered:

### Computational Thinking

Problem → Input → Process → Decision → Output

### MATLAB

variables
operators
sqrt()
disp()
input()
if–elseif–else
roots()
plot()

### Engineering Thinking

Predict → Calculate → Visualize → Verify → Interpret

### AI-Assisted Programming

Ask → Examine → Improve → Verify

The main lesson is:

Human defines and judges the problem.
MATLAB performs the computation.
AI assists and expands the solution.
The engineer remains responsible for the result.

# CURIOSITY BOX — WHAT COMES NEXT?

Our first program uses only three numbers:

a, b, c.

But civil engineers work with:

- hundreds of concrete-strength measurements;

- thousands of survey observations;

- decades of rainfall records;

- hourly water levels;

- traffic data; and

- large sensor datasets.

How can MATLAB store and process all these values efficiently?

That brings us to:

# CHAPTER 2

# From One Number to Thousands: Vectors, Matrices, and Engineering Data