# CHAPTER 5

# LINEAR BEST FITTING AND USER-DEFINED FUNCTIONS IN MATLAB

MATLAB Programming with AI | Undergraduate Engineering

# Chapter Learning Outcomes

Students will organize engineering observations; derive least-squares regression; solve coefficients manually and using MATLAB; create reusable functions; assess residuals, RMSE and R²; distinguish interpolation from extrapolation; and calculate confidence and prediction intervals.

# 5.1 Engineering Problem: Rainfall and Runoff

Engineers often seek empirical relationships between observed variables. Here, five hypothetical rainfall events provide paired total rainfall depths P and corresponding direct-runoff depths R. Both are expressed in millimetres. These illustrative values are not field-validated.

| Event | Rainfall P (mm) | Runoff R (mm) |
| --- | --- | --- |
| 1 | 10 | 4 |
| 2 | 20 | 7 |
| 3 | 30 | 11 |
| 4 | 40 | 14 |
| 5 | 50 | 18 |

P = [10;20;30;40;50];

R = [4;7;11;14;18];

figure; scatter(P,R,70,"b","filled"); grid on;

xlabel("Rainfall, P (mm)"); ylabel("Runoff depth, R (mm)");

Figure 5.1. Observed rainfall and runoff depths.

Observed events are plotted as individual points without connecting lines.

# 5.2 Linear Relationship

Assume a linear model relating predicted runoff to rainfall:

Here a is the intercept (mm), b is the slope (dimensionless), and the hat indicates a predicted response. The intercept need not have a physical interpretation at zero rainfall.

# 5.3 Least-Squares Principle

## 5.3.1 Residuals

A residual is the difference between an observed and predicted runoff depth:

Positive residuals mean observed runoff exceeds predicted runoff; negative residuals mean the model overpredicts.

## 5.3.2 Sum of Squared Errors

Least squares selects coefficients minimizing the sum of squared residuals:

The optimum satisfies both derivative conditions:

# 5.4 Manual Calculation of Regression Coefficients

Differentiating SSE yields the normal equations:

| P | R | P² | PR |
| --- | --- | --- | --- |
| 10 | 4 | 100 | 40 |
| 20 | 7 | 400 | 140 |
| 30 | 11 | 900 | 330 |
| 40 | 14 | 1600 | 560 |
| 50 | 18 | 2500 | 900 |
| 150 | 54 | 5500 | 1970 |

With n = 5, substitution gives:

Subtracting 30 times the first equation from the second gives 1000b = 350, hence b = 0.35 and a = 0.30.

# 5.5 Solving Regression Using Matrices

The normal equations can be expressed as a matrix system, connecting this chapter to Chapter 4:

n = length(P);

A = [n sum(P); sum(P) sum(P.^2)];

d = [sum(R); sum(P.*R)];

c = A\d; a = c(1); b = c(2);

A numerically preferable direct least-squares approach avoids explicitly forming the normal equations:

X = [ones(length(P),1), P];

c = X\R; a = c(1); b = c(2);

P_line = linspace(min(P),max(P),100);

scatter(P,R,70,"b","filled"); hold on;

plot(P_line,a+b*P_line,"r-","LineWidth",2);

legend("Observed runoff",sprintf("R = %.2f + %.2fP",a,b));

xlabel("Rainfall, P (mm)"); ylabel("Runoff depth, R (mm)"); grid on;

Figure 5.3. Observed points and fitted regression equation.

# 5.6 MATLAB Built-in Fitting

polyfit calculates polynomial coefficients; polyval evaluates the fitted polynomial. For a straight line, degree = 1. Coefficients are returned in descending powers: [b a].

p = polyfit(P,R,1);

b = p(1); a = p(2);

R_pred = polyval(p,P);

| P (mm) | Observed R (mm) | Predicted R (mm) |
| --- | --- | --- |
| 10 | 4 | 3.8 |
| 20 | 7 | 7.3 |
| 30 | 11 | 10.8 |
| 40 | 14 | 14.3 |
| 50 | 18 | 17.8 |

The normal-equation solution, direct X\R least-squares solution, and polyfit agree for this dataset (up to numerical rounding).

# 5.7 Creating a User-Defined Function

A function accepts inputs, performs a calculation in its own workspace, and returns outputs. Save the following as myLinearFit.m:

function [a,b,R_pred] = myLinearFit(P,R)

P = P(:); R = R(:);

if numel(P) ~= numel(R) || numel(P) < 2

error("Input vectors must have equal lengths >= 2.");

end

if any(~isfinite(P)) || any(~isfinite(R))

error("Inputs must contain finite numbers.");

end

if numel(unique(P)) < 2

error("P must contain two distinct values.");

end

X = [ones(numel(P),1), P];

c = X\R;

a = c(1); b = c(2);

R_pred = a + b*P;

end

Call the function from a separate script in the same folder:

[a,b,R_pred] = myLinearFit(P,R);

fprintf("a = %.4f, b = %.4f\n",a,b);

# 5.8 Model Evaluation

Model evaluation quantifies residual magnitude and explained variation.

A high R² does not guarantee reliable predictions, particularly for a small synthetic dataset.

e = R-R_pred;

SSE = sum(e.^2);

RMSE = sqrt(mean(e.^2));

SST = sum((R-mean(R)).^2);

R2 = 1-SSE/SST;

figure; scatter(P,e,70,"b","filled"); hold on;

yline(0,"r--"); grid on;

Figure 5.6. Residuals versus rainfall.

MATLAB also supports rmse(R_pred,R). With Statistics and Machine Learning Toolbox, fitlm(P,R) provides mdl.Rsquared.Ordinary and mdl.RMSE. The latter divides SSE by residual degrees of freedom (n − 2), yielding about 0.3162 mm rather than 0.2449 mm.

# 5.9 Engineering Interpretation and Prediction

The slope of 0.35 means each additional millimetre of rainfall corresponds to approximately 0.35 mm additional fitted runoff within the calibration range. For P = 35 mm (interpolation):

For P = 80 mm (extrapolation):

Extrapolation beyond the observed 10–50 mm rainfall range requires particular caution because hydrological behavior may change.

# 5.10 Confidence and Prediction Intervals

A confidence interval describes uncertainty in the estimated mean response. A prediction interval describes uncertainty for an individual new observation and is generally wider. For P₀, the classical intervals are:

At P₀ = 35 mm, the predicted mean is 12.55 mm. The approximate 95% CI for mean runoff is 12.04–13.06 mm, and the approximate 95% PI for an individual observation is 11.42–13.68 mm.

mdl = fitlm(P,R);

P_line = linspace(10,50,100)';

[R_mean,CI] = predict(mdl,P_line,"Alpha",0.05,...

"Prediction","curve");

[~,PI] = predict(mdl,P_line,"Alpha",0.05,...

"Prediction","observation");

figure; h1 = scatter(P,R,60,"b","filled"); hold on;

h2 = plot(P_line,R_mean,"r-","LineWidth",2);

h3 = plot(P_line,CI(:,1),"g--");

plot(P_line,CI(:,2),"g--","HandleVisibility","off");

h4 = plot(P_line,PI(:,1),"m:");

plot(P_line,PI(:,2),"m:","HandleVisibility","off");

legend([h1 h2 h3 h4],"Observed runoff",...

"R = 0.30 + 0.35P","95% confidence interval",...

"95% prediction interval"); grid on;

Figure 5.7. Fitted line with 95% confidence and prediction intervals.

These intervals assume an appropriate linear model with independent, normally distributed errors and constant error variance. Five observations are insufficient to assess these assumptions confidently. A 95% confidence procedure has 95% long-run coverage; it does not assign 95% probability to a fixed parameter after observing the sample.

# 5.11 Student Assignment: Load and Deflection

A structural engineer measures the following hypothetical applied loads and vertical deflections. Fit the empirical model δ = a + bF within the measured range.

| Observation | Load F (kN) | Deflection δ (mm) |
| --- | --- | --- |
| 1 | 2 | 0.8 |
| 2 | 4 | 1.7 |
| 3 | 6 | 2.5 |
| 4 | 8 | 3.4 |
| 5 | 10 | 4.1 |
| 6 | 12 | 5.1 |

Required tasks: (1) scatter plot; (2) manual normal-equation derivation; (3) matrix solution; (4) polyfit/polyval; (5) myLinearFit function; (6) SSE, RMSE and R²; (7) residual plot; (8) predict at 7 and 15 kN and identify interpolation/extrapolation; (9) 95% confidence and prediction intervals; (10) interpret limitations.

| Assessment component | Weight |
| --- | --- |
| Manual derivation | 15% |
| Matrix calculations | 15% |
| Built-in fitting and plotting | 15% |
| User-defined function | 15% |
| Model evaluation | 15% |
| Intervals | 15% |
| Engineering interpretation | 10% |

# Chapter Summary

The chapter connects engineering observations to mathematical models, least-squares derivation, MATLAB matrix computation, built-in fitting, reusable functions, error assessment, uncertainty intervals, and engineering interpretation. Students should distinguish numerical agreement from physical validity and avoid uncritical extrapolation.

Key MATLAB commands: scatter, plot, sum, mean, \, polyfit, polyval, function, sqrt, rmse, fitlm, predict.